Control of Grobo by Software via I²C

v1.0 16042022 – CaptainHawk, eda

This manual can be a start for software developers to control the Grobo with a Raspberry Pi or an Arduino via I²C and some GPIO-ports. Attached you can find datasheets of the chips and sample codes for control via I²C.

We found out that most of the Grobo is controlled by two PWM-chips and one GPIO-chip. There is no microcontroller at the mainboard, the firmware to control these three chips comes via the WiFi-chip from the cloud.

Table of Contents

- Board Descriptions

  - 1 – LED Board

  - 2 – Upper Board

  - 3 – Lower Board

Board Description - LED Board

The LED board holds 6 branches of LEDs.

- 7 RB (PWM Channel 1)

- 7 B (PWM Channel 2)

- 6 C (PWM Channel 3)

- 10 R (PWM Channel 4)

- 7 W (PWM Channel 5)

- 9 DR (PWM Channel 7)

In original RB, B and C were at 85% duty and the other 3 were at 100% when we measured, don't know which stage of grow.

In the middle of the board is a LM75 I²C Temp Sensor at address 0x49.

The board is connected to the upper board with a 26-pin flat-ribbon-cable.

Board Description - Upper Board

The upper board holds:

- WiFi-chip (which is already soldered out in my imges)

- 6 led-driver controlled by one pwm-chip PCA9685 at I²C address 0x74

- Power Converters from 24V to12V, 5V and 3.3V

- Connectors for:

  - 24V power in (P1)

  - Door Lock (LOCK)

  - User Button (UBTN)

  - Exhaust Fan (FAN)

  - Led Lamp (26-pin flat-ribbon-cable)

  - Temp/Hum-sensor in the rear panel (DHTI) (Not I²C)

  - 8-pin flat-ribbon-cable-connector to the lower board

  - USB (don't now how to connect to this, maybe the most simple possibility to connect to the board and control it)

Door Lock:

Can be triggered by pulling the pin with the dot of transistor next to the connector HIGH (3v3). It is wired to the wifi-chip to pin 4 from the lower left angle. Ground line is switched by the transistor.

Exhaust Fan:

Is always on I think, can be switched off by pulling the pin with the dot of the transistor Q4 LOW. Ground line is switched by the transistor.

Exh Fan

to lower board

24V in

Door Lock

Temp/Hum

User Button

Door Lock

PWM chip

led lamp

C

DR

R

B

W

RB

my way to connect to i²c

Board Description - Lower Board

The lower board is connected by an 8-pin fat-ribbon-cable to the upper board.

It holds:

- one pwm-chip PCA9685 at I²C address 0x76 to control pumps an fans.

- one gpio-chip MCP23008 at I²C address 0x26 for the input of the 3 water level sensors and 5 buttons that could be soldered in for control of the 5 nutrient-pumps.

- Power Converter from 5V to 3.3V

- LLC ADM3260 to convert the 3v3 I²C to 5v I²C for the PH-sensor

- PH and EC sensors

  - Atlas Scientific pH and EC stamps (red and green ‘mini’ boards by the BNC connectors)

- Connectors for

  - lower water level sensor (WLL) (GPIO Channel 5)

  - middle water level sensor (WLM) (GPIO Channel 6)

  - high water level sensor (WLH) (GPIO Channel 7)

  - nutrient pump N1 (PHU) (PWM Channel 7)

  - nutrient pump N2 (PHD) (PWM Channel 1)

  - nutrient pump N3 (A) (PWM Channel 2)

  - nutrient pump N4 (B) (PWM Channel 3)

  - nutrient pump N5 (CMG) (PWM Channel 4)

  - drain pump (DRAIN) (PWM Channel 6)

  - fill pump (FILL) (PWM Channel 5)

  - rear fan (FAN1) (PWM Channel 9)

  - rear fan (FAN2) (PWM Channel 8)

  - 8-pin flat-ribbon-cable-connector to the upper board

  - 5 solder-pins for switches which are not soldered in (GPIO Channels 0 - 4)

LLC chip

GPIO chip

to upper board

PWM chip